Terms of Reference (TOR)

for

Installation of 20 Solar‑Powered Submersible Pump Systems

Locations: Communities under Botiaghata and Dumuria Upazilas of Khulna District, and Rampal Upazila of Bagerhat District

Scope of Work

Supply of Solar Submersible Pump Motor Set & Controller (350W-750W)

Supply of Solar PV Modules (1420Watt; 10‑year warranty)

Drilling of tubewells (site‑specific depth: up to 1000–1500 ft)

Supply of all required PVC/GI pipes, filters, clamps, and accessories

Construction of an external structure for a 2,000‑liter overhead water tank

Complete installation, testing, and commissioning of all systems

Transportation and on‑site delivery for all materials

- Solar Powered Submersible Pump Specification 1000-1500 feet

Part A: Drilling & Installation Section

| Sl. No | Particulars | Brand | "Quantity (per unit)" | Total Quantity for 20 sets | Cost per unit | Total Cost for 20 Sets |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4" PVC Pipe (Class D) 6.0-6.90mm | NPoly | 160 feet | 3200 feet |  |  |
| 2 | 1.5" PVC Pipe (Class E) 3.10-3.70mm | NPoly | 1250 rft | 25000 rft |  |  |
| 3 | 1.5" PVC Filter (10 No) | NPoly | 40 feet | 800 feet |  |  |
| 4 | 1.25" Thread Pipe 3.7mm | NPoly | 8 pcs | 160 pcs |  |  |
| 5 | 1.25" Socket | NPoly | 8 pcs | 160 pcs |  |  |
| 6 | 70 T Gum | Taiwan 500 gms | 3 pcs | 60 pcs |  |  |
| 7 | SS wire | China | 180 feet | 3600 feet |  |  |
| 8 | Thread tep | NPoly | 5 pcs | 100 pcs |  |  |
| 9 | U Clamp | China | 2 pcs | 40 pcs |  |  |
| 10 | Tie Clamp | Ericson | 1 dozen | 20 dozen |  |  |
| 11 | 1.25"X1" GI Elbow | Any | 1 pc | 20 pcs |  |  |
| 12 | 1" Pipe Nipple | Any | 1 pc | 20 pcs |  |  |
| 13 | 4"X1.25" SS Cap | Any | 1 pc | 20 pcs |  |  |
| 14 | Marble | Any | 300 pcs | 6000 pcs |  |  |
| 15 | 1/4 Stone | Any | 12 cft | 240 cft |  |  |
| 16 | Cement | Any | 10 kg | 200 kg |  |  |
| 17 | Brush | China | 3 pcs | 60 pcs |  |  |
| 18 | 1.5" GI Pipe Lite (Top Pipe) | Any | 5 feet | 100 feet |  |  |
| 19 | Test Drilling |  | 1500 feet | 30000 feet |  |  |
| 20 | Drilling Charges |  | 1 unit | 20 unit |  |  |
| 21 | Soil Sample Collection |  | 1 unit | 20 unit |  |  |
| 22 | Water Test | DPHE | 1 unit | 20 unit |  |  |
|  | Section Total |  |  |  |  |  |

Part B: External Structure Section

| 23 | External Structure building for 2000 liters water tank and Solar panels | As per the attachment | 1 unit | 20 units |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Section Total |  |  |  |  |  |

Part C: Electrical Section

| 24 | Auto Solar Pump Motor Set & Controller (Built in) Capacity: 350 Watts to 750 Watts, based on the site depth Voltage: 108V Dc Max Head: 65 m (223 Feet) Water Flow: 3-5 m3/H Dia: 2 inches to 3 inches Delivery:1.0 to 1.5 inches Country Origin: China Warranty: minimum 2 years |  | 1 set | 20 sets |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | Solar PV Capacity: 1420w Maximum Power Output: 1420W Brand: Longi/fortune/equivalent Warranty: minimum 10 years |  | pcs | pcs |  |  |
| 26 | DB Box & Other accessories |  | 1 set | 20 sets |  |  |
| 27 | 70/76 -BRB Cable |  | 10 m | 200 m |  |  |
| 28 | Electrical Installation Charges |  | 1 set | 20 Sets |  |  |
|  | Section Total |  |  |  |  |  |

Part D: Logistic Section

| 29 | Transport cost |  | 1 set | 20 sets |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Section Total |  |  |  |  |  |
| 30 | Grand Total (A+B+C+D) |  |  |  |  |  |

Note:

- 1.5" PVC Pipe (Class E), actual price will be fixed on a per-site running feet (rft) basis.

- Drilling Charges will be fixed on a per-site running feet (rft) basis

- Solar-powered auto submersible pump will be finalized based on drilling depth

- Solar Powered Submersible Pump Specification up to1000 feet

Part A: Drilling & Installation Section

| Sl. No | Particulars | Brand | "Quantity (per unit)" | Total Quantity for 20 sets | Cost per unit | Total Cost for 20 Sets |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4" PVC Pipe (Class D) 6.0-6.90mm | NPoly | 160 feet | 3200 feet |  |  |
| 2 | 1.5" PVC Pipe (Class D) 2.50-2.70mm | NPoly | 750 rft | 15000 rft |  |  |
| 3 | 1.5" PVC Filter (10 No) | NPoly | 40 feet | 800 feet |  |  |
| 4 | 1.25" Thread Pipe 3.7mm | NPoly | 8 pcs | 160 pcs |  |  |
| 5 | 1.25" Socket | NPoly | 8 pcs | 160 pcs |  |  |
| 6 | 70 T Gum | Taiwan 500 gms | 3 pcs | 60 pcs |  |  |
| 7 | SS wire | China | 180 feet | 3600 feet |  |  |
| 8 | Thread tep | NPoly | 5 pcs | 100 pcs |  |  |
| 9 | U Clamp | China | 2 pcs | 40 pcs |  |  |
| 10 | Tie Clamp | Ericson | 1 dozen | 20 dozen |  |  |
| 11 | 1.25"X1" GI Elbow | Any | 1 pc | 20 pcs |  |  |
| 12 | 1" Pipe Nipple | Any | 1 pc | 20 pcs |  |  |
| 13 | 4"X1.25" SS Cap | Any | 1 pc | 20 pcs |  |  |
| 14 | Marble | Any | 300 pcs | 6000 pcs |  |  |
| 15 | 1/4 Stone | Any | 12 cft | 240 cft |  |  |
| 16 | Cement | Any | 10 kg | 200 kg |  |  |
| 17 | Brush | China | 3 pcs | 60 pcs |  |  |
| 18 | 1.5" GI Pipe Lite (Top Pipe) | Any | 5 feet | 100 feet |  |  |
| 19 | Test Drilling |  | 1000 feet | 20000 feet |  |  |
| 20 | Drilling Charges |  | 1 unit | 20 unit |  |  |
| 21 | Soil Sample Collection |  | 1 unit | 20 unit |  |  |
| 22 | Water Test | DPHE | 1 unit | 20 unit |  |  |
|  | Section Total |  |  |  |  |  |

Part B: External Structure Section

| 23 | External Structure building for 2000 liters water tank and Solar panels | As per the attachment | 1 unit | 20 units |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Section Total |  |  |  |  |  |

Part C: Electrical Section

| 24 | Auto Solar Pump Motor Set & Controller (Built in) Capacity: 350Watt to 750Watt, based on site depth Voltage: 108V Dc Max Head: 65 m (223 Feet) Water Flow: 3-5 m3/H Dia: 2 inches to 3 inches Delivery:1.0 to 1.5 inches Country Origin: China Warranty: minimum 2 years |  | 1 set | 20 sets |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | Solar PV Capacity: 1420w Maximum Power Output: 1420W Brand: Longi/fortune/equivalent Warranty: minimum 10 years |  | pcs | pcs |  |  |
| 26 | DB Box & Other accessories |  | 1 set | 20 sets |  |  |
| 27 | 70/76 -BRB Cable |  | 10 m | 200 m |  |  |
| 28 | Electrical Installation Charges |  | 1 set | 20 Sets |  |  |
|  | Section Total |  |  |  |  |  |

Part D: Logistic Section

| 29 | Transport cost |  | 1 set | 20 sets |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Section Total |  |  |  |  |  |
| 30 | Grand Total (A+B+C+D) |  |  |  |  |  |

Note:

- 1.5" PVC Pipe (Class D), actual price will be fixed on a per-site running feet (rft) basis.

- Drilling Charges will be fixed on a per-site running feet (rft) basis

- Solar-powered auto submersible pump will be finalized based on drilling depth

General terms and conditions:

- Pricing of PVC Pipe:
The cost of 1.5" PVC Pipe (Class D & Class E) will be determined on a per-site running feet (rft) basis, depending on the actual requirement at each installation site.

- Drilling Charges:
Drilling costs will be finalized on a per-site running feet (rft) basis, based on the actual depth required.

- Submersible Pump:

Solar-powered auto submersible pump will be finalized based on drilling depth

- Split Procurement Rights:
The procuring entity reserves the right to award different parts of the work to different vendors depending on quality, pricing, delivery capacity, and other relevant terms and conditions.

- Tax Deduction:
All applicable taxes will be deducted from the vendor’s quoted price as per prevailing Government rules and regulations.

- Section-wise Quotation Option:
Vendors may submit quotations for specific sections or for the full package, according to their capacity and interest.

- Submission of Quotation:
Quotations must be submitted to the Programme Manager–Field, SOS Children’s Village Khulna, Sher-E-Bangla Road, Gollamari, Khulna-9100, in a sealed envelope clearly marked:
“Quotation for installing 20 sets of solar-powered submersible pumps.”
Alternatively, quotations may be emailed to: sossckln@sos-bangladesh.org
Deadline: On or before 25 March 2026, 05:00 PM.

- Installation Site Details:
Detailed locations of the installation sites will be communicated to the selected vendor prior to signing the formal agreement.

- Site Supervision:
A dedicated site engineer assigned by the organization will regularly monitor the progress and quality of the work at each site.

- Inspection of Goods:
All supplied materials may be subject to inspection by the concerned technical personnel before acceptance, as deemed necessary by the relevant department.

- Quality Testing:
A quality test of the supplied pipes will be conducted at KUET (Khulna University of Engineering & Technology) to ensure compliance with required standards.

- Quantity Variation:
The final quantity of materials or work may be increased or decreased at the time of formal agreement, depending on actual field needs.

- Payment Terms:
Payment will be made via crossed cheque or EFTN, after:

- deduction of applicable TDS,

- successful installation at each site,

- verification and approval through a Satisfactory Installation Report issued by the concerned technical authority, and

- compliance with the terms of the formal agreement.

Quotation Submission Requirements

Company Profile

Valid Trade License, VAT Certificate, TIN Certificate

Item‑wise unit rate and total cost as per BOQ

Warranty details for all components

Delivery, installation & commissioning timeline

Quotation validity (minimum 30 days)

Any additional terms or conditions

Submission Deadline:

Date: 25 March 2026

Time: 05:00 pm

Submission Address:

Programme Manager-Field

SOS Children’s Village Khulna

113, Sher-E-Bangla Road, Gollamari, Khulna

Or Email to the following address Directly with the

Subject: Installation of 20 Solar‑Powered Submersible Pump Systems

sossckln@sos-bangladesh.org

and cc to soscvkln@sos-bangladesh.org

Contact: 01925100320

Evaluation Criteria

Compliance with technical specifications

Competitive pricing

Warranty and after‑sales support

Previous relevant experience

Delivery & installation schedule

The procuring authority reserves the right to accept or reject any quotation without assigning any reason.

ANNEX

evwn¨K ÷ªvKPv‡ii weeib

- mvegvwm©ej cv¤ú I 2000 wjUvi cvwbi U¨vsK cÖwZ¯’vcb ÷ªvKPvit

mvegvwm©ej cv‡¤úi Dci 8 dzU ˆ`N©¨ I 5 dzU cÖ‡¯’i 5 BwÂ B‡Ui Mv_wbi GKwU Ni n‡e hvi D”PZv 7 dzU| D³ N‡ii Dc‡i I wb‡P 5x5 BwÂi Aviwmwm wjswUb _vK‡e| Pvi †Kvbvq 10 BwÂi wcjvi Ges ‰`‡N©¨ 5 AšÍi Av‡ivI `yBwU wcjvi w`‡q 5 BwÂi †`qvj n‡e| D³ 5x5 dz‡Ui ¯’vb evwj ev gvwU w`‡q c~Y© K‡i GK B‡Ui cøv÷vi n‡e hvi Dci 2000 wjUv‡ii cvwbi U¨vsK em‡e| gUv‡ii Dci 5x3 dz‡Ui GKwU XvjvB ¯øvf n‡e| cvwbi U¨vsK n‡Z †d¬vi n‡Z 2 dzU D”PZvq 3 wU cvwbi Kj em‡e| K‡ji wb‡P 1 B‡Ui 2 dzU cøv÷vi n‡e hvi GK mvB‡W cvwb bvgvi †Wªb n‡e| Pvi cvk cø÷vi K‡i cÖwZôvb KZ„K wba©vwiZ is K‡i mvg‡bi cv‡k cv_‡ii eªvwÛs †cøU emv‡Z n‡e|

- ‡mvjvi c¨v‡bj cÖwZ¯’vcb ÷ªvKPvit

wb‡¤œv³ ÷ªvKPvi 1 bs G D‡jøwLZ ÷ªvKPv‡ii Dci 1 dzU Mfxi ch©šÍ XvjvB K‡i cÖwZ¯’vcb Kiv n‡e|

ANNEX